Lab Task 1: Design an IP Address Scheme

- The network 172.16.10.0/24 was divided into eight subnets, as outlined below:

- The value of the new subnet mask is 255.255.255.224.

- 30 usable hosts exist per subnet.

| Subnet Number | Network Address | Usable Host Address Range | Broadcast Address |
| --- | --- | --- | --- |
| 1 | 172.16.10.0 | 172.16.10.1 - 172.16.10.30 | 172.16.10.31 |
| 2 | 172.16.10.32 | 172.16.10.33 - 172.16.10.62 | 172.16.10.63 |
| 3 | 172.16.10.64 | 172.16.10.65 - 172.16.10.94 | 172.16.10.95 |
| 4 | 172.16.10.96 | 172.16.10.97 - 172.16.10.126 | 172.16.10.127 |
| 5 | 172.16.10.128 | 172.16.10.129 - 172.16.10.158 | 172.16.10.159 |
| 6 | 172.16.10.160 | 172.16.10.161 - 172.16.10.190 | 172.16.10.191 |
| 7 | 172.16.10.192 | 172.16.10.193 - 172.16.10.222 | 172.16.10.223 |
| 8 | 172.16.10.224 | 172.16.10.225 - 172.16.10.254 | 172.16.10.255 |

Lab Task 2: Implement VLANs and Trunk
(Listed commands were executed on S1-Office1 and S2-Office1.)

- en
conf t
vlan 10
name Management
exit
vlan 20
name Marketing
exit
vlan 30
name Accounting
exit
vlan 100
name Native
exit

- int range fa0/1-10
switchport mode access
switchport access vlan 10
exit
int range fa0/11-20
switchport mode access
switchport access vlan 20
exit
int range fa0/21-24
switchport mode access
switchport access vlan 30
exit

  - On S1-Office1: 
int gi0/2
switchport mode trunk
switchport trunk native vlan 100
exit
do sho vlan brief
do sho int trunk

  - On S2-Office1: 
int gi0/1
switchport mode trunk
switchport trunk native vlan 100
exit
do sho vlan brief
do sho int trunk

- int range fa0/1-24
switchport nonegotiate
exit
do wr

Lab Task 3: Assign IP Addresses

- Address 172.16.10.1 (from subnet 1) was assigned to the R3 / S1-Office3 link.
On R3:
en
conf t
int gi0/0
ip address 172.16.10.1 255.255.255.224
desc To S1-Office3
no shut
exit

- Address 172.16.10.33 (from subnet 2) was assigned to the R3 / S1-Office2 link.
On R3: 
int gi0/1
ip address 172.16.10.33 255.255.255.224
desc To S1-Office2
no shut
exit

- For the R1 / R2 WAN link, addresses from subnet 3 were used.  Address 172.16.10.65 was assigned on the R1 side, and address 172.16.10.66 was assigned on the R2 side.
On R1:
en
conf t
int se0/0/1
ip addr 172.16.10.65 255.255.255.224
desc To R2
no shut
ex
On R2:
en
conf t
int se0/0/1
ip addr 172.16.10.66 255.255.255.224
desc To R1
no shut
ex

- For the R1 / R3 WAN link, addresses from subnet 4 were used.  Address 172.16.10.97 was assigned on the R1 side, and address 172.16.10.98 was assigned on the R3 side.
On R1:
int se0/0/0
ip addr 172.16.10.97 255.255.255.224
desc To R3
no shut
ex
do wr
On R3:
int se0/0/0
ip addr 172.16.10.98 255.255.255.224
desc To R1
no shut
ex

- For the R2 / R3 WAN link, addresses from subnet 5 were used.  Address 172.16.10.129 was assigned on the R2 side, and address 172.16.10.130 was assigned on the R3 side.
On R2:
int se0/0/0
ip addr 172.16.10.129 255.255.255.224
desc To R3
no shut
ex
do wr
On R3:
int se0/0/1
ip addr 172.16.10.130 255.255.255.224
desc To R2
no shut
ex
do wr

- The last usable addresses on subnet 6 were assigned to end devices on VLAN 10 on the Office 1 LAN.  The first usable address in subnet 6, 172.16.10.161, was set as the default gateway.

  - CEO1: 172.16.10.189

  - CEO2: 172.16.10.190

- The last usable addresses on subnet 7 were assigned to end devices on VLAN 20 on the Office 1 LAN.  The first usable address in subnet 7, 172.16.10.193, was set as the default gateway.

  - Copywriter1: 172.16.10.221

  - Copywriter2: 172.16.10.222

- The last usable addresses on subnet 8 were assigned to end devices on VLAN 30 on the Office 1 LAN.  The first usable address in subnet 8, 172.16.10.225, was set as the default gateway.

  - Dialer1: 172.16.10.253

  - Dialer2: 172.16.10.254

- IP addresses were assigned to end devices in Offices 2 and 3:

  - The last usable addresses on subnet 2 were assigned to end devices on the Office 2 LAN.  The first usable address in subnet 2, 172.16.10.33, was set as the default gateway.

    - Employee 1: 172.16.10.60

    - Employee 2: 172.16.10.61

    - Guest: 172.16.10.62

  - The last usable addresses on subnet 1 were assigned to end devices on the Office 3 LAN.  The first usable address in subnet 1, 172.16.10.1, was set as the default gateway (as configured in step 1).

    - Email Server: 172.16.10.28

    - Syslog / NTP Server: 172.16.10.29

    - Admin: 172.16.10.30

| IP Address Usage Summary |  |
| --- | --- |
| Subnet | Use |
| 1 | Office 3 LAN |
| 2 | Office 2 LAN |
| 3 | R1 / R2 WAN link |
| 4 | R1 / R3 WAN link |
| 5 | R2 / R3 WAN link |
| 6 | VLAN 10 (Management) |
| 7 | VLAN 20 (Marketing) |
| 8 | VLAN 30 (Accounting) |

Lab Task 4: Configure R1 for Inter-VLAN Routing
(Listed commands were executed on R1, unless otherwise stated.)

- en
conf t
int gi0/0
no shut
ex

- (The commands below cover lab steps 2 through 4)
int gi0/0.10
encapsulation dot1q 10
ip addr 172.16.10.161 255.255.255.224
ex
int gi0/0.20
encapsulation dot1q 20
ip addr 172.16.10.193 255.255.255.224
ex
int gi0/0.30
encapsulation dot1q 30
ip addr 172.16.10.225 255.255.255.224
ex

- do sho run

- On S1-Office1:
en
conf t
int gi0/1
switchport mode trunk
switchport trunk native vlan 100
no shut
ex

- On S1-Office1: do sho int trunk

- I was able to successfully ping Copywriter1 and Dialer1 from the CEO1 PC.

Lab Task 5: Secure Switch Physical Ports
(Listed commands were executed on both S1-Office1 and S2-Office1, unless otherwise stated)

- en
conf t
int range fa0/1-24
switchport port-security
switchport port-security violation restrict

- switchport port-security mac-address sticky
switchport port-security maximum 1
ex

- do sho port-sec

- On S1-Office1:
int range fa0/2-10
shut
ex
int range fa0/12-20
shut
ex
int range fa0/22-24
shut
ex
do wr
On S2-Office1:
int range fa0/2-10
shut
ex
int range fa0/12-20
shut
ex
int range fa0/22-24
shut
ex
int gi0/2
shut
ex
do wr

Lab Task 6: Configure OSPF
(Listed commands were executed on R1, R2, and R3, unless otherwise stated)

- en
conf t
int se0/0/0
no shut
ex
int se0/0/1
no shut
ex

- On R3 only: 
int range gi0/0-1
no shut
ex

- router ospf 1
auto-cost reference-bandwidth 1000
On R1:
router-id 1.1.1.1
network 172.16.10.64 0.0.0.31 area 0
network 172.16.10.96 0.0.0.31 area 0
network 172.16.10.160 0.0.0.31 area 0
network 172.16.10.192 0.0.0.31 area 0
network 172.16.10.224 0.0.0.31 area 0
On R2:
router-id 2.2.2.2
network 172.16.10.64 0.0.0.31 area 0
network 172.16.10.128 0.0.0.31 area 0
On R3:
router-id 3.3.3.3
network 172.16.10.96 0.0.0.31 area 0
network 172.16.10.128 0.0.0.31 area 0
network 172.16.10.32 0.0.0.31 area 0
network 172.16.10.0 0.0.0.31 area 0

- On R1:
passive-int gi0/0
passive-int gi0/0.10
passive-int gi0/0.20
passive-int gi0/0.30
On R3:
passive-int gi0/0
passive-int gi0/1

- ex
ex
sho ip route ospf
sho ip ospf neighbor
sho ip protocols
wr

Lab Task 7: Initial and Security Settings for Network Devices
(Listed commands were executed on all routers and switches)

- en
conf t
username Admin password ACDC1973

- line console 0
login local
exit

- enable password beatles1960

- service password-encryption

- banner motd #Access to this device is for authorized personnel only!#
ex
wr
ex

Lab Task 8: Secure Remote Access
(Listed commands were executed on R1, R2, and R3)

- ip domain-name Cyber.com

- crypto key generate rsa
2048

- ip ssh version 2

- line vty 0 4
login local
motd-banner
transport input ssh
exit

- ex
sho ip ssh
sho run
wr

- Using the command ssh -l Admin IP-Address, I was able to successfully SSH into the routers.

Lab Task 9: Full Connectivity Test

- The configuration of IP addresses, subnet masks, default gateways, and wildcard masks were verified to be correct on all devices.

- From the Admin PC, I was able to successfully ping CEO1 and Employee1.

- From the Employee2 PC, I was able to successfully ping Copywriter1 and Dialer1.

Lab Task 10: Extended ACL
(The below commands were executed on R3.)

- en
conf t
access-list 100 deny ip host 172.16.10.62 host 172.16.10.29
access-list 100 permit ip any any
int gi0/1
ip access-group 100 in
ex

- ex
sho access-lists
sho run
wr

- From the Guest PC, I was able to ping the email server but was not able to ping the NTP server.